Driver Installation

Step1：Uninstall the driver (note that if it is a newly installed system and the Cirrus Logic Amplifier driver has not been installed, please skip to Step 2)：

After decompressing Cirrus-Uninstall-ver_2. zip (enter 666666 if you need the decompression password), right-click on the CS3551_Uninstall.bat file and select "Run as administrator" to complete the uninstallation.

Step2，View current device driver information:

Open the CMD command line and enter: pnputil /enum-devices /class media

If the machine has not installed the Cirrus Logic Amplifier driver, it will display:

At this point, there is no Cirrus Logic Awesome Speaker Amps information, indicating that the machine has not installed or has successfully uninstalled our driver.

Step3，Installing Drivers:

- Find the file name corresponding to the drivers in the base driver: csaudio.inf， Right click and select Install, as shown in the figure:

- After installation, you can view the driver information through the command line above:

C:\Windows\system32>pnputil /enum-devices /class media

At this point, the Cirrus Logic Amazing Speaker Amps information appears in the device driver enumeration, indicating that the base driver (displayed here as oem1-INF, which may not be the same name for other machines) has been successfully installed.

- Continue to find the file name 'csaudioext.aYANEO_0819', right-click and select 'install', as shown in the figure:

- All drivers have been installed, and the driver information can be viewed through the previous command line;

C:\Windows\system32>pnputil /enum-devices /class media

Compared to before, there is an additional Extension Driver Names (oem20. info) information, indicating that our extension driver has been successfully installed.

At this point, the following information can be viewed from the Device Manager:

At this point, it indicates that the Cirrus Logic Amplifier driver has been successfully installed

Note: This article only applies to WHQL Certified drivers, and non WHQL Certified drivers are not within the scope of this article.