Precautions:

1. Do not cut off the power during the BIOS update process, otherwise the BIOS update will be incomplete, resulting in failure to boot.

2. After updating the BIOS, you need to cut off the AC power supply, and then turn it on again.

- After decompressing the compressed package, copy the entire EFI folder to the root directory of the U disk (the format of the U disk must be FAT32).

- Plug in the U disk, then turn it on, press F7 to display the interface as shown in the figure, select the U disk by pressing the up and down keys on the keyboard, and then press the Enter key to start from the U disk.

- The interface as shown in the figure appears after the option to start from the U disk.

- The interface shown in the above figure appears, indicating that the BIOS update has been completed, and the BIOS update process will take about 5 minutes.